AMIRIS Input Data Requirements

A.  Electricity Demand

| Data item | Unit | Resolution | What it is |
| --- | --- | --- | --- |
| Hourly load | MW (or MWh/h) | Hourly - 8,760 values/yr | Total German electricity demand to be met each hour. |

B.  Conventional Fleet

Needed separately for each of: Nuclear, Lignite, Hard coal, Natural gas, Oil.

| Data item | Unit | Resolution | What it is |
| --- | --- | --- | --- |
| Installed capacity | MW | One value/yr (or per capacity change) | Total capacity of that fuel type's fleet. |
| Efficiency | % (LHV basis) | One value or range per fuel type | Share of fuel energy converted to electricity. |
| Typical block size | MW per plant | One value per fuel type | Size of a single plant, used to split the fleet into individual units. |
| Variable operating cost | EUR/MWh | One value per fuel type | Non-fuel cost of running the plant one more hour. |
| CO2 emissions factor | t CO2/MWh | One value per fuel type | CO2 released per unit of electricity produced. |
| Outage / availability | % unavailable | Hourly if possible; monthly/seasonal will work also | Share of the fleet offline for maintenance or breakdown. |

C.  Renewable Fleet

Needed separately for each of: Solar PV, Wind onshore, Wind offshore, Biomass, Run-of-river hydro, Other renewables.

| Data item | Unit | Resolution | What it is |
| --- | --- | --- | --- |
| Installed capacity | MW | One value/yr (or per capacity change) | Maximum possible output; split by subsidy vintage/scheme if available. |
| Generation profile | MW/h or capacity factor (0-1) | Hourly - 8,760 values/yr (most critical item) | Actual weather-driven output shape, not just the capacity total. |
| Subsidy / support scheme | EUR/MWh or scheme parameters | One value per technology/vintage | Fixed tariff or market top-up paid to the plant owner. |

D.  Storage (Batteries, Pumped Hydro, etc.)

| Data item | Unit | Resolution | What it is |
| --- | --- | --- | --- |
| Power capacity | MW | One value/yr | Maximum charge/discharge rate. |
| Energy capacity | MWh | One value/yr | Total amount storable. |
| Round-trip efficiency | % | One value | Share of stored energy recovered on discharge. |
| Number of operators | count | One value, if available | Single pooled operator vs. several competing ones. |

E.  Fuel and Carbon Prices

| Data item | Unit | Resolution | What it is |
| --- | --- | --- | --- |
| Hard coal price | EUR/MWh (thermal) | Monthly or quarterly | Cost of coal delivered to the plant, per unit of energy content. |
| Natural gas price | EUR/MWh (thermal) | Monthly or quarterly | Cost of gas delivered to the plant, per unit of energy content. |
| Oil price | EUR/MWh (thermal) | Monthly or quarterly | Cost of oil delivered to the plant, per unit of energy content. |
| CO2 allowance price | EUR/tonne CO2 | Monthly or quarterly | EU ETS carbon permit price. |