EDITORIAL NOTE (remove before submission): Draft candidate appendix, condensed from AMIRIS_Germany2027_Progress_Report.pdf/.docx into a single table, one row per phase, in formal register, for review before deciding whether to include it in the thesis. If adopted, renumber as Appendix A (or per the HSN template's own appendix convention), add a cross-reference from the relevant point in Chapter 3/4/5, and add it to the Table of Contents and (if used) the List of Tables.

Appendix A: Experimental Phase Log

Table A.1 summarises, in chronological order, every distinct test or build phase carried out in the course of developing and validating the AMIRIS Germany 2027-2029 scenario described in Chapters 3-5, including phases whose result was negative or null. It is provided for transparency and reproducibility; the full narrative account, including the reasoning behind each design decision, is given in the relevant section of Chapters 3-6. Phase 52 is reported for completeness as an investigative finding only and was not adopted into the thesis's own methodology (see Section 6.x).

Table A.1: Condensed log of all 52 experimental phases.

| Phase | Intervention / test | Result |
| --- | --- | --- |
| 1 | Resolve five identified gaps in Brainpool's supplied 2027 input data (outages/must-run, offshore wind profile, renewable subsidy rates, battery storage duration, cross-border trade). | All five resolved using AMIRIS's own reference data or independent research; cross-border trade deferred to a dedicated later build. |
| 2 | Build V1: full conventional and renewable fleet with a single BDEW household load profile applied to total demand, no cross-border import. | Market cleared, but shortage pricing (3,000 EUR/MWh) occurred in 15.9% of hours. |
| 3 | Produce build documentation (input provenance, methodology Q&A). | Documentation completed; no simulation result. |
| 4 | Build V2: replace the single demand profile with four separately shaped components (base load, heat pumps, electrolysis, e-mobility). | Shortage hours fell from 15.9% to 7.5%. |
| 5 | Add an ImportTrader sized to Brainpool's stated net-import figure, shaped by 2023 cross-border flow data, to both V1 and V2. | Shortage hours fell further, but only 7-12% of the intended import volume actually cleared the market. |
| 6 | Obtain Brainpool's real 2027 hourly price forecast and compare it against all four builds. | AMIRIS's mean price was 3-7x Brainpool's; on the 92-95% of hours without a shortage, price levels were already close. |
| 7 | Diagnose the cause of the mean-price gap by inspecting a real shortage hour agent-by-agent. | 64% of shortage hours had zero import available; the import schedule was copied from an unrelated year. |
| 8 | Replace the shaped import series with a flat, market-determined hourly ceiling; calibrate the ceiling against Brainpool's price. | V1 shortage hours 11.8% to 0.83%, mean price 400 to 75 EUR/MWh; V2 shortage hours 5.4% to 0.03%, mean price 220 to 57 EUR/MWh. |
| 9 | Test Brainpool's own reported 2009 weather-year basis for wind/solar, in place of AMIRIS's default. | Mixed: closest-ever average price match (within 0.51 EUR/MWh) when combined with the import fix, but day-to-day pattern-matching did not improve. |
| 10 | Split the AMIRIS-Brainpool comparison by month, hour-of-day, and weekday/weekend to localise the residual gap. | Found a approximately 39 EUR/MWh systematic weekday-vs-weekend gap, traced to unaligned calendar days in the demand-building step. |
| 11 | Shift the 2023 source data by five days so weekdays align with the 2027 calendar. | The weekday/weekend gap narrowed by two-thirds; correlation and mean error both improved on trusted hours. |
| 12 | Replace generic climate-normal temperature with real 2009 hourly DWD temperature for the heat-pump demand component. | No material effect (heat pumps are under 3% of total demand) - reported as a null result. |
| 13 | Inspect a real midday hour agent-by-agent to diagnose the midday price gap. | Found differential renewable curtailment behaviour across subsidy types, and that the model has no mechanism to export surplus power abroad. |
| 14 | Build and test an experimental one-way virtual export device. | Never activated across the simulated year; confirmed AMIRIS, as configured, cannot represent cross-border export without structural changes. |
| 15 | Verify Brainpool's published weather-year and model-scope assumptions; rebuild the import price as an 11-country real blend. | 2009 weather-year assumption confirmed correct; Brainpool's own model covers 30 countries. The 11-country blend gave a small, genuine improvement. |
| 16 | Test whether Germany's 2027 public holidays explain the residual price gap. | An initial apparent pattern did not survive controlling for season and normal day-to-day variation - a genuine null result. |
| 17 | Identify the closest real weather-matched demand-source year to 2009 (candidates: 2015-2019, 2023) and rebuild demand on it. | 2016 was the best match (2023 the worst). Rebuilding on 2016 reduced shortage hours further; the trustworthy excl-shortage fit stayed flat. |
| 18 | Research the regulatory origin of AMIRIS's 3,000 EUR/MWh default shortage price. | Confirmed as the real EU/ACER harmonised day-ahead price cap in force November 2017-May 2022, covering AMIRIS's own 2019 reference year. |
| 19 | Test the effect of raising the shortage price to the current real EU cap (5,000 EUR/MWh). | Bias improved slightly, but all-hours correlation worsened (0.439 to 0.317); the excl-shortage comparison was unaffected either way. Not adopted. |
| 20 | Rebuild electrolysis as a price-responsive GenericFlexibilityTrader agent instead of flat demand. | Shortage hours fell (4 to 1); all-hours correlation improved (0.439 to 0.553); excl-shortage correlation essentially unchanged. |
| 21 | Apply the same price-responsive pattern to e-mobility (smart charging). | Shortage hours reached zero for the first time in the project; trustworthy correlation improved (0.640 to 0.647), a verified, non-artefactual result. |
| 22 | Test Reservoir Hydro's power-rating sensitivity (doubling/tripling its real 1.54 GW figure). | Modest, consistent gains at each step, but the power-cap constraint (hit 35% of the year) never fully cleared even at 3x. Reported as a finding, not adopted. |
| 23 | Re-run the granular month/hour/weekday breakdown; investigate a newly observed July-October seasonal bias. | The midday gap remained dominant despite added flexibility. Five candidate causes for the seasonal bias were ruled out; traced to the known single-zone-vs-30-country structural limitation. |
| 24 | Validate out-of-sample on 2028 and 2029 using Brainpool's newly supplied multi-year data, with no re-tuning. | Price level held up across both years; correlation weakened with distance from the calibration year (0.647, 0.446, 0.349). A real AMIRIS leap-year limitation was also found and documented. |
| 25 | Isolate the out-of-sample correlation loss: test the import-ceiling value and the demand-source year as candidate causes. | A full ceiling sweep found smaller ceilings give better 2028/2029 correlation but at a large shortage-hour cost; the demand-source year was ruled out as a driver. |
| 26 | Adopt a smaller (20,000 MW) import ceiling for 2028/2029, following the Phase 25 trade-off. | Excl-shortage correlation improved sharply (to 0.705/0.698); adopted as a deliberate, documented exception to reusing the 2027 configuration unchanged. |
| 27 | Re-examine the Phase 26 adoption using all-hours statistics, not only excl-shortage. | All-hours correlation and bias both worsened at smaller ceilings. 30,000 MW was re-adopted as the default for both years; the 20,000 MW results remain preserved as documented evidence. |
| 28 | Investigate a reversed weekday/weekend correlation pattern found in the 2028/2029 re-run of the Phase 10 breakdown. | Found and fixed a genuine bug: dropping 29 February from the leap-year demand source misaligned every day from March onward (84% of the year). Clean improvement for 2027 and 2029, no trade-off. |
| 29 | Investigate a negative price floor as a lighter alternative to full market coupling, at the supervisor's suggestion. | Confirmed the -500 EUR/MWh floor is hard-coded in AMIRIS's compiled engine. No tested floor value, including the supervisor's own suggested +10 EUR/MWh, improved correlation. Confirmed dead end. |
| 30 | Test a real, literature-sourced plant start-up/cycling cost for conventional generators. | Output was byte-identical to the baseline across all 8,760 hours; the parameter is computed internally by AMIRIS but never reaches the single-zone bid-price logic. Confirmed dead end. |
| 31 | Sweep lignite's negative-bidding markup band (-60 to -10 EUR/MWh) against Germany's real negative-price frequency. | Bias, MAE, and negative-hour frequency improved modestly and consistently as the band narrowed; the trustworthy excl-shortage correlation stayed flat. A working lever that does not move the key metric. |
| 32 | Scope a market-coupling build: study AMIRIS's own two-zone template; confirm real cross-border capacity data availability. | Confirmed as a genuine, working AMIRIS mechanism. Scope decision: build one aggregate Rest-of-Europe zone first. |
| 33 | Source real Rest-of-Europe demand/capacity data (ENTSO-E, then Eurostat after an ENTSO-E outage). | Confirmed a genuine multi-day ENTSO-E platform outage; substituted real Eurostat 2023 data for 9 of 10 neighbouring countries. Switzerland flagged as a confirmed data gap. |
| 34 | Build the first working market-coupling scenario, using a placeholder transmission capacity. | Excl-shortage correlation rose to 0.7394 with 1 shortage hour - the first evidence coupling was worth pursuing, pending a real transmission figure. |
| 35 | Backfill real cross-border flow data and Switzerland's figures once the ENTSO-E outage resolved. | Real 2023 bilateral flow data obtained for both directions of the DE-ROE link; the Swiss data gap closed. |
| 36 | Replace the Phase 34 placeholder with the real flow-derived transmission capacity. | Shortage hours rose to 26, but excl-shortage correlation barely moved (0.7394 to 0.7288), confirming the Phase 34 gain was not a placeholder-capacity artefact. |
| 37 | Add real Rest-of-Europe storage (pumped/reservoir hydro) and renewable subsidy realism, mirroring Germany's own build. | Mean price 67.45 EUR/MWh (Brainpool: 68.04); excl-shortage bias -2.90, MAE 16.69, correlation 0.72. The project's best result and the thesis's reference build. |
| 38 | Extend the Phase 37 build unchanged to 2028 and 2029 (frozen Rest-of-Europe methodology). | Excl-shortage correlation held up well in both new years (0.7181, 0.7518); shortage hours grew with distance from 2027 (7, 17, 76). |
| 39 | Test scaling the transmission ceiling to each year's own demand growth as a fix for rising out-of-sample shortage hours. | Made 2028 worse. Rejected. |
| 40 | Test MarketCoupling's configurable per-iteration energy-shift limit as a candidate cause. | Confirmed already effectively unlimited by default; tested directly and also rejected. |
| 41 | Test whether Rest-of-Europe's own storage runs physically dry during out-of-sample shortage hours. | Storage never exceeded approximately 25% of its own discharge power during these hours. Rejected. |
| 42 | Disaggregate France from the aggregate Rest-of-Europe pool as its own named zone (star topology). | Shortage hours rose sharply (7 to 167); bias worsened to +9.65. A genuine, informative negative result against unnecessary disaggregation. |
| 43 | Disaggregate all 10 real neighbouring countries (star topology), testing the Phase 42 prediction at full scale. | Found and fixed a genuine AMIRIS engine bug (sub-scale dispatch failure). Result improved on the France-only pilot (shortage 167 to 120) but still fell short of Phase 37's simpler approach. |
| 44 | Rebuild the Rest-of-Europe zone on real 2024 data as a robustness check, isolating the data year as the only variable. | Close to, but not better than, Phase 37 (correlation 0.7168 to 0.7066; shortage hours improved slightly, 7 to 5). |
| 45 | Instrument AMIRIS's own source code at every exit point in the coupling algorithm; test against all 76 real 2029 shortage hours. | Hypothesis of a hidden engine bug refuted: every stop traced to a legitimate cause (72% real supply exhaustion, 18% transmission ceiling, 9% a correctness guard). |
| 46 | Source a real ENTSO-E ERAA growth trajectory for Rest-of-Europe demand/capacity/transmission, replacing the frozen 2024 snapshot. | Mixed: 2028 shortage hours improved (17 to 10); 2029 worsened (76 to 89), traced to a genuine real-world decline in dispatchable capacity. |
| 47 | Test an alternative shortage-pricing convention (ShortagePriceMethod=LastSupplyPrice); diagnose the root cause of every remaining shortage hour. | All-hours correlation rose from 0.3003 to 0.7167 with excl-shortage metrics unchanged. 4 of 7 shortage hours found transmission-ceiling-bound; the other 3 traced to forecaster blindness in Rest-of-Europe's storage agent. |
| 48 | Add 12 real bilateral Rest-of-Europe transmission links (24 directional) on top of the Phase 43 star topology. | Shortage hours fell further (120 to 95); correlation stayed flat (0.7311 to 0.7297). Mesh outperforms star, but not Phase 37's aggregate approach. |
| 49 | Test a real, temperature-matched alternative demand-shape year against the Phase 37 reference. | Every trusted metric moved the wrong way (shortage 7 to 26; correlation 0.7168 to 0.7104). Rejected. |
| 50 | Sweep 16 real years of wind-offshore weather data; test the single most promising candidate. | Every trusted metric again moved the wrong way (shortage 7 to 12; correlation 0.7168 to 0.7124). Rejected. |
| 51 | Test a real alternative run-of-river hydro weather year, completing the renewable weather-year sweep. | A near-wash, marginally worse on every metric (shortage 7 to 8). Completes a three-for-three negative result confirming Phase 37's choices were not fortuitous. |
| 52 | Patch AMIRIS's own Java source code directly to test whether Rest-of-Europe's storage could be made aware of Germany's shortage price (investigative only, not adopted into the thesis methodology). | Best variant: shortage hours 7 to 6, bias -2.90 to -0.95; correlation/MAE remained slightly worse than the headline build in every variant tested, bounded by Rest-of-Europe storage's finite daily energy budget. |